FMT2D

Fast Marching Tomography 2D

Theory and User Manual

Version: v1.0
Compiler environment: GNU Fortran (gfortran) 12.3.0
Updated: 2026-10-27

Contents

1. Software Overview

2. Software Architecture and Modules

3. Forward Modeling: Fast Marching Method (FMM)

4. Ray Tracing

5. Inversion: LSQR Joint Inversion

6. Three Smoothing Types

7. Input and Output File Formats

8. Compilation and Installation

9. Example Scripts

9.1 run_vel_checkerboard.py

9.2 run_joint_checkerboard_moho.py

9.3 run_joint_checkerboard_sed.py

9.4 run_pgss_checkerboard.py

10. Inversion Parameter Tuning Guide

11. Troubleshooting

1. Software Overview

1.1 Introduction

FMT2D (Fast Marching Tomography 2D) is a two-dimensional seismic tomography package that uses the Fast Marching Method (FMM) to solve the eikonal equation for traveltime forward modeling and inversion of marine seismic first-arrival and reflected phases. The software supports joint inversion of velocity structure and reflector interface geometry, including the Moho and the base of the sedimentary layer, and is suitable for velocity-model building from active-source marine seismic data.

1.2 Main Features

FMT2D provides the following main features:

FMM-based two-dimensional traveltime forward modeling for P waves.

Backward ray tracing from receivers to sources, supporting multi-segment reflected/refracted paths.

LSQR iterative inversion with joint or alternating velocity–interface updates.

Three smoothing regularization types: Laplacian, Gaussian, and Hybrid.

Five supported phases: Pg, PsedP, PmP, Pg_ss, and Pg_ss2.

Five interfaces: water surface, seafloor, base of the sedimentary layer, Moho, and model bottom.

Four complete example scripts.

1.3 Supported Phase Types

| Phase | ID | Meaning | Phase-weight parameter |
| --- | --- | --- | --- |
| Pg | 1 | Refracted wave in the solid medium | default 1.0 |
| PsedP | 2 | Reflection from the base of the sedimentary layer | phase_wt_psed |
| PmP | 3 | Moho reflection | phase_wt_pmp |
| Pg_ss | 4 | Source-side water-layer multiple of Pg | phase_wt_pgss |
| Pg_ss2 | 5 | Receiver-side water-layer multiple of Pg | phase_wt_pgss |

1.4 Interfaces and Regions

FMT2D uses five interfaces to divide the model into six regions：

| Interface No. | Name | Description |
| --- | --- | --- |
| 1 | Water surface | z = 0 (fixed) |
| 2 | Seafloor | Provided by the topo array in the smesh file (fixed) |
| 3 | Base of the sedimentary layer | Provided by sed_file (invertible) |
| 4 | Moho | Provided by refl_file (invertible) |
| 5 | Model bottom | zpos_rel(nz) + maxval(topo) (fixed) |

| Region No. | Name | Upper/lower interfaces | Use |
| --- | --- | --- | --- |
| 1 | Water layer | 1 ↔ 2 | Propagation in water |
| 2 | Sedimentary layer | 2 ↔ 3 | Propagation within sediments |
| 3 | Crust | 3 ↔ 4 | Propagation within the crust |
| 4 | Mantle | 4 ↔ 5 | Propagation within the mantle |
| 5 | Combined: sediment + crust + mantle | 2 ↔ 5 | Used for Pg, Pg_ss, and Pg_ss2 |
| 6 | Combined: sediment + crust | 2 ↔ 4 | Used for PmP |

2. Software Architecture and Modules

2.1 Program Components

FMT2D consists of two independent executable programs：

| Program | Source file | Function |
| --- | --- | --- |
| fmt2d | fmt2d_main.f90 | P-wave forward modeling (traveltimes and ray paths) |
| invert2d | invert2d_main.f90 | Standalone inversion (joint velocity–interface inversion) |

2.2 Source-File Structure

| Source file | Main function |
| --- | --- |
| mod_fmt2d.f90 | Type definitions, global variables, explicit interfaces |
| propagate2d.f90 | Core FMM propagation algorithm |
| rays2d.f90 | Ray tracing (backward stepping integration) |
| frechet2d.f90 | Fréchet derivative computation |
| inversion2d.f90 | LSQR inversion core module |
| read_tomo2d.f90 | Input-file readers (smesh, refl, geom) |
| fmt2d_main.f90 | Forward-modeling main program |
| invert2d_main.f90 | Inversion main program |

2.3 Module Hierarchy

The main Fortran modules are：

type_definitions2d       (derived types and default initialization)
       │
       ├── global_variables2d   (global parameters and model entities)
       │
       ├── interface_definitions2d  (explicit interface declarations)
       │
       └── mod_fmt2d            (aggregates the three modules above)
              │
              └── mod_fmt2d_nointerfaces  (type/global definitions without explicit interfaces, used by lower-level modules)
                     │
                     ├── mod_prop2d       (FMM propagation)
                     ├── mod_rays2d       (ray tracing)
                     ├── mod_frechet2d    (Fréchet derivatives)
                     └── mod_inversion2d  (LSQR inversion)

3. Forward Modeling: Fast Marching Method (FMM)

3.1 Eikonal Equation

FMT2D forward modeling is based on the two-dimensional eikonal equation:

|∇T(x,z)| = 1 / V(x,z)

where T(x,z) is the traveltime field and V(x,z) is the medium velocity. FMM solves this equation to obtain the first-arrival traveltime field.

3.2 FMM Core Algorithm

FMM uses a narrow-band method, a binary-heap priority queue, and upwind finite differences to solve the eikonal equation. It has a single-sweep structure with approximately O(N log N) complexity.

Node-state classification：

| State | Identifier | Meaning |
| --- | --- | --- |
| ALIVE | node_status = 1 | Traveltime already determined |
| TRIAL | node_status = 0 | In the narrow band, traveltime pending |
| FAR | node_status = -1 | Far field, not yet reached |

Main algorithm loop：

1. Initialize the heap with the active/alive nodes.

2. Remove the minimum-traveltime node from the heap and finalize it.

3. Find connected nodes (four-neighborhood or interface neighbors).

4. Update connected-node traveltimes using the upwind difference scheme.

5. If the new traveltime is smaller than the old value, update the heap structure or convert a FAR node to TRIAL.

6. Repeat steps 2–5 until the heap is empty.

3.3 Upwind Difference Scheme

For regular interior nodes, the code assembles a local quadratic equation in T from upwind finite-difference terms and solves for the smaller root. Second-order terms are used where available. If the discriminant is negative or the stencil degenerates, the update falls back to a first-order scheme.

3.4 Interface-Node Handling

Nodes near interfaces are treated specially:

Interface neighbors are obtained through ccell_from_inode mapping.

Interface-node traveltimes are interpolated from adjacent cells.

For multi-type grids, S-wave nodes in the water layer are masked when the velocity is below 0.01.

For nodes with suspect gradients (suspect_time_gradient), the gradient is recomputed by least-squares fitting.

4. Ray Tracing

4.1 Backward Tracing Strategy

FMT2D traces rays backward from the receiver to the source. Each ray consists of multiple sections; each section is associated with a traveltime field corresponding to a region. The sections are processed in reverse order until the source is reached.

4.2 Ray-Section Structure

Each ray is described by the Tpath2d type:

sequence：region/interface sequence defining the propagation order.

tf_sequence：time-field sequence.

vtype_sequence：wave-type sequence (P/S).

refraction：flag (0 = reflection segment, 1 = refraction segment).

4.3 Stepping Integration

A single section is traced by stepping integration based on the traveltime gradient ∇T:

ray direction = -∇T / |∇T|

Core control parameters:

| Parameter | Default | Meaning |
| --- | --- | --- |
| maxpoints | 5000 | Maximum number of ray points per section |
| dlmax | 0.1 * dx * v_average | Maximum step length |
| v_average | 5.0 | Initial average velocity (km/s) |

4.4 Interface-Crossing Handling

When a ray section reaches its ending interface:

The intersection point is located by linear interpolation.

The validity of the crossing is checked with the crossing_ok flag.

For a reflection segment, the ray is reflected and enters the next time field.

For a refraction segment, the code searches for a head-wave seed node（n_headwave, seed_node）

5. Inversion: LSQR Joint Inversion

5.1 Inversion Framework

FMT2D inversion uses an LSQR iterative framework with damping and smoothing regularization. It supports joint or alternating inversion of velocity and interfaces.

Objective function:

minimize  ||A*δm - d||^2  +  λ_damp * ||δm||^2  +  λ_smooth * ||L*δm||^2

where:
  A      = sparse Fréchet derivative matrix
  δm      = model perturbation vector (velocity + interface)
  d      = traveltime residual vector
  L      = smoothing operator (Laplacian or Gaussian)
  λ_damp = damping coefficient
  λ_smooth = smoothing coefficient

5.2 Alternating Inversion Modes

The inversion mode is controlled by alt_inv_mode:

| alt_inv_mode | Mode | Description |
| --- | --- | --- |
| 0 | Joint inversion | Velocity and interfaces updated jointly |
| 1 | Velocity only | Interfaces fixed, only velocity updated |
| 2 | Interface only | Velocity fixed, only interfaces updated |
| -1 | Automatic alternating | First three of every four iterations update velocity; the fourth updates interfaces; cycle repeats |

5.3 Parameter Scaling and Covariance

mvscale: velocity parameters are scaled by 1/velocity_init; interface parameters are scaled by refl_weight.

cm (prior covariance): velocity cm = (0.05 * velocity_init)^2; interface cm = 1.0.

ecmi (damping weight): ecmi = lam_damp / cm for velocity, or lam_damp_iface / cm for interfaces.

5.4 Inversion Iteration Flow

Initialization: compute mvscale, cm, and ecmi.

Initial forward run: compute initial residuals in residuals_initial.dat.

Iteration loop (iter = 1, niter):

a. Determine the update type for this iteration (joint / velocity / interface).

b. Run forward modeling, count valid data, and reject outliers with |res| > inv_crit_chi.

c. Assemble the Fréchet derivative matrix and residual vector.

d. Call LSQR to solve for the model perturbation.

e. Apply perturbation constraints (max_rel_pert limits a single relative update to ≤30%).

f. Track the best model (best_rms, best_iter).

Output the best model and residuals.

5.5 Fréchet Derivatives

Fréchet derivatives describe the sensitivity of traveltime to model parameters:

Velocity perturbation: dT/dV is integrated along the ray path by ray_partials2d.

Interface perturbation: dT/dz_interface is computed from the interaction between the traveltime gradient and the interface normal.

Only velocity nodes with active = .true. participate in inversion; water-layer nodes are excluded.

Each active node is assigned a global index inv_idx, and multiple interfaces are numbered sequentially.

5.6 Water-Layer Traveltime Correction

Each ray has an independent water-layer correction term, water_time. Precomputed entry-point data (entry_point_data) ensure consistency between forward modeling and inversion. Entry-point information is stored in entry_points.dat.

6. Three Smoothing Types

FMT2D controls smoothing regularization through smooth_type. Three smoothing methods are supported and can be applied separately to velocity and interfaces in Hybrid mode:

| smooth_type | Name | Velocity smoothing | Interface smoothing | Characteristics |
| --- | --- | --- | --- | --- |
| 1 | Laplacian | Second-order Laplacian | Second-order Laplacian | Preserves linear perturbations and sharp boundaries |
| 2 | Gaussian | Gaussian（R_h × R_v） | Gaussian（R_h） | Anisotropic; correlation lengths can be controlled |
| 3 | Hybrid | Laplacian | Gaussian（R_h） | Sharp velocity, smooth interfaces |

6.1 Laplacian Smoothing (smooth_type=1)

The conventional finite-difference Laplacian operator is used:

L*T(i,j) = T(i+1,j) + T(i-1,j) + T(i,j+1) + T(i,j-1) - 4*T(i,j)

Characteristics:
- Preserves linear perturbations and sharp boundaries.
- Suitable for synthetic checkerboard tests.

6.2 Gaussian Smoothing (smooth_type = 2)

Separable Gaussian smoothing is used (horizontal R_h × vertical R_v):

Precomputed CSR sparse matrices:
  gs_h_rowptr / gs_h_colind / gs_h_values (horizontal smoothing)
  gs_v_rowptr / gs_v_colind / gs_v_values (vertical smoothing)

Correlation-length parameters:
  CORR_X     = horizontal velocity correlation length (recommended 10 km)
  CORR_Z     = vertical velocity correlation length (recommended 5 km)
  CORR_X_IFACE = horizontal interface correlation length (recommended 10 km)

Characteristics:
- Anisotropic smoothing with independent horizontal and vertical control.
- Effectively reduces ray footprints.
- Suitable for real-data inversion.

6.3 Hybrid Smoothing (smooth_type = 3)

Laplacian smoothing is used for velocity, and Gaussian smoothing is used for interfaces:

Velocity smoothing: Laplacian (sharpness-preserving).
Interface smoothing: Gaussian (gsi_h_rowptr, gsi_h_colind, gsi_h_values).

Characteristics:
- Velocity structure retains details.
- Interfaces are smoothed to avoid spurious spikes.
- Suitable for joint velocity–Moho inversion.

7. Input and Output File Formats

7.1 Velocity-Grid File (*.smesh)

The smesh file uses a token-based format (line breaks are flexible):

nx nz vwater vair
xpos(1) xpos(2) ... xpos(nx)
topo(1) topo(2) ... topo(nx)
zpos_rel(1) zpos_rel(2) ... zpos_rel(nz)
vel(1,1) vel(1,2) ... vel(1,nz)
vel(2,1) vel(2,2) ... vel(2,nz)
...
vel(nx,1) vel(nx,2) ... vel(nx,nz)

Field descriptions:
  nx, nz       = number of grid points
  vwater       = water velocity (km/s)
  vair         = air velocity (placeholder)
  xpos         = horizontal coordinates (km)
  topo         = seafloor depth (km)
  zpos_rel     = depth relative to the seafloor (km)
  vel(ix,iz)   = velocity (km/s), stored by column

7.2 Interface Files (*.refl, *.dat)

x1 z1
x2 z2
...
xn zn

Each line contains the (x, z) coordinates of one node. Nodes must be ordered by increasing x.

7.3 Geometry File (geom.dat)

nobs                              ! number of sources
S  obs_x1 obs_z1 nrec1           ! source 1: label S, coordinates, number of receivers
R  rec_x  rec_z  ray_type  t_obs  unc   ! receiver 1
R  rec_x  rec_z  ray_type  t_obs  unc   ! receiver 2
...
S  obs_x2 obs_z2 nrec2           ! source 2
R  ...

Field descriptions:
  nobs     = number of sources
  obs_x/z  = source coordinates
  nrec     = number of receivers for this source
  ray_type = phase ID (1=Pg, 2=PsedP, 3=PmP, 4=Pg_ss, 5=Pg_ss2)
  t_obs    = observed traveltime (s)
  unc      = picking uncertainty (s); currently stored but not used for weighting in the inversion

7.4 fmt2d.in (Forward-Modeling Input)

Line 1: smesh_vp_file      velocity-grid file
Line 2: refl_file          Moho interface file
Line 3: sed_file           base-of-sedimentary-layer interface file
Line 4: geom_file          observation geometry file
Line 5: write_rays         whether to output ray paths (.true./.false.)
Line 6: arrivals_file      output traveltime file name
Line 7: obs_file           observed traveltime file name
Line 8: rays_prefix        ray-file prefix

7.5 invert2d.in (Inversion Input)

Line  1: smesh_vp_file        velocity grid
Line  2: refl_file            Moho interface
Line  3: sed_file             base of the sedimentary layer
Line  4: geom_file            observation geometry
Line  5: niter_inv            number of inversion iterations
Line  6: lam_damp_in          velocity damping
Line  7: lam_smooth_in        velocity smoothing
Line  8: inv_crit_chi_in      residual threshold (outlier rejection)
Line  9: lsqr_max_iter_in     maximum inner LSQR iterations
Line 10: target_pert_in       target perturbation
Line 11: lam_damp_iface_in    interface damping
Line 12: lam_smooth_iface_in  interface smoothing
Line 13: refl_weight_in       reflection weight
Line 14: phase_wt_pmp_in      PmP phase weight
Line 15: phase_wt_pgss_in     Pg_ss/Pg_ss2 phase weight
Line 16: phase_wt_psed_in     PsedP phase weight
Line 17: corr_x_in            horizontal correlation length
Line 18: corr_z_in            vertical correlation length
Line 19 (optional): smooth_type, target_dd, iface_step_ratio, max_rel_pert
Line 20 (optional): alt_inv_mode
Line 21 (optional): corr_x_iface
Line 22 (optional): iface_dx_target

7.6 frechet.in (Fréchet Control File)

Line 1: do_frechet          switch (0 = do not compute, 1 = compute)
Line 2: ngrids              number of velocity grids participating in inversion
Line 3: grid_id(1:ngrids)   grid-ID list
Line 4: vtype(1:ngrids)     wave-type list (1 = P, 2 = S)
Line 5: nifaces             number of interfaces participating in inversion (0 = velocity only)
Line 6: iface_id(1:nifaces) interface-ID list (3 = sediment bottom, 4 = Moho)

Example (velocity-only inversion):
1
1
1
1
0

Example (joint velocity + Moho inversion):
1
1
1
1
1
4

7.7 Main Output Files

| File | Content |
| --- | --- |
| arrivals.dat | Computed traveltimes |
| obs_times.dat | Observed traveltimes |
| rays_src_src*.dat | Ray paths (one file per source) |
| entry_points.dat | Water-layer entry-point information |
| vgrid_1_vtype1.smesh | Final inverted velocity model |
| iface_4.refl | Final inverted Moho interface (if inverted) |
| model_iter*.dat | Model snapshot at each iteration |
| residuals_initial.dat | Initial residuals |
| residuals_final.dat | Final residuals |
| ray_density_*.dat | Ray density |
| invert2d.log | Inversion log |
| invert_log_latest.txt | Latest inversion-log summary |

8. Compilation and Installation

8.1 Environment Requirements

| Component | Requirement |
| --- | --- |
| Compiler | GNU Fortran (gfortran) >= 11 |
| Build tool | GNU Make |
| Operating system | Linux / WSL / macOS |
| Python (examples only) | Python 3.8+, with numpy, matplotlib, scipy |

8.2 Compilation Steps

# Enter the project directory
cd /path/to/FMT

# Clean old files and rebuild
make clean && make

# Compiled executables
#   bin/fmt2d     forward-modeling program
#   bin/invert2d  inversion program

# Run the executables in a working directory containing the corresponding input files (fmt2d.in for forward modeling and invert2d.in for inversion).
./bin/fmt2d < test/fmt2d.in
./bin/invert2d < test/invert2d.in

8.3 Makefile Description

Key Makefile settings:

FC = gfortran
FFLAGS = -O2 -fbacktrace -cpp

# Two targets
all: fmt2d invert2d

# Forward dependencies
FWD_OBJS = mod_fmt2d.o propagate2d.o rays2d.o read_tomo2d.o fmt2d_main.o

# Inversion dependencies
INV_OBJS = mod_fmt2d.o propagate2d.o rays2d.o read_tomo2d.o \
           frechet2d.o inversion2d.o invert2d_main.o

9. Example Scripts

FMT2D provides four complete Python example scripts. All scripts are located in the project root and are run as:

python3 run_<name>.py

Overview of the four scripts:

| Script | Working directory | Smoothing type | Inversion content | Runtime |
| --- | --- | --- | --- | --- |
| run_vel_checkerboard.py | example_inv_vel_only | Laplacian | Vp only | ~5 min |
| run_joint_checkerboard_moho.py | example_inv_joint_moho | Laplacian | Vp + Moho | ~10 min |
| run_joint_checkerboard_sed.py | example_inv_joint_sed | Laplacian | Vp + Sed | ~30 min |
| run_pgss_checkerboard.py | example_inv_pgss | Laplacian | Vp only | ~40 min |

9.1 run_vel_checkerboard.py — Velocity Checkerboard Test

Function:

Velocity-only checkerboard test using Pg + PmP. Interfaces are fixed at their true values to verify pure velocity recovery.

Synthetic-model parameters:

| Parameter | Value | Description |
| --- | --- | --- |
| NX × NZ | 343 × 35 | Grid size |
| X 范围 | 0 - 114 km | Horizontal extent |
| CB_DX / CB_DZ | 18.0 / 5.0 km | Checkerboard cell size |
| CB_AMP | 0.05 | Checkerboard perturbation amplitude (±5%) |
| OBS 数 | 14 | Number of ocean-bottom seismometers |
| Pg offset | 3-50 km | Pg offset range |
| PmP offset | 5-50 km | PmP offset range |

Key inversion parameters:

| Parameter | Value | Description |
| --- | --- | --- |
| SMOOTH_TYPE | 1 (Laplacian) | Laplacian smoothing |
| LAM_DAMP | 0.15 | Velocity damping |
| LAM_SMOOTH | 0.3 | Velocity smoothing weight |
| CORR_X / CORR_Z | 18.0 / 5.0 | Correlation lengths |
| INV_NITER | 15 | Number of inversion iterations |
| VEL_ONLY | True | Velocity-only inversion (nifaces = 0) |
| Inversion phases | {1, 3} | Pg and PmP |

Main outputs (example_inv_vel_only/):

model_iter16.dat — best inverted model

vgrid_1_vtype1.smesh — final inverted velocity

step1_init_model.pdf — initial model

step2_true_model.pdf — true model

step4_forward.pdf — forward-modeling rays

step5_vel_inversion.pdf — velocity-inversion comparison

step6_rms_convergence.pdf — RMS convergence curve

step8_ray_density.pdf — ray density

Key result metrics:

Velocity correlation (all solid)

Velocity correlation (shallow, above Moho)

Perturbation difference RMS (%)

Run example:

cd /mnt/d/FMT
python3 run_vel_checkerboard.py

# Typical output
Velocity correlation (all solid): 0.6668
Velocity correlation (shallow, above Moho): 0.9109
Perturbation difference RMS: 1.6475%

9.2 run_joint_checkerboard_moho.py — Joint Velocity–Moho Checkerboard Test

Function:

Joint inversion of velocity structure and the Moho interface using Pg + PmP. The initial Moho is flat, and the test verifies recovery of an undulating Moho.

Synthetic-model characteristics:

True Moho: base = 10.5 km, AMP = 0.72, WAVELENGTH = 40.0 km (undulating)

Initial Moho: flat at 10.5 km (to be inverted)

PmP offset: 5–30 km (shorter offsets focusing on Moho reflections)

Inverted interface: iface = 4 (Moho)

Key inversion parameters:

| Parameter | Value | Description |
| --- | --- | --- |
| SMOOTH_TYPE | 1 (Laplacian) | Laplacian smoothing |
| LAM_DAMP | 0.25 | Velocity damping (higher, more stable) |
| LAM_SMOOTH | 0.4 | Velocity smoothing weight |
| LAM_DAMP_IFACE | 0.1 | Interface damping |
| LAM_SMOOTH_IFACE | 1.5 | Interface smoothing weight |
| INV_NITER | 20 | Number of inversion iterations |
| TARGET_PERT | 0.015 | Target perturbation (1.5%) |
| VEL_ONLY | False | Joint inversion (nifaces = 1, iface = 4) |

Main outputs (example_inv_joint_moho/):

iface_4.refl — final inverted Moho interface

model_iter21.dat — best model containing velocity + interface

step5_joint_velocity.pdf — joint velocity-inversion result

step5_moho_recovery.pdf — Moho-recovery comparison

step6_rms_convergence.pdf — RMS convergence curve

Key result metrics:

Velocity correlation (shallow, above Moho)

Moho RMSE: init vs inverted (km)

RMS convergence curve

Run example:

cd /mnt/d/FMT
python3 run_joint_checkerboard_moho.py

# Typical output:
Velocity correlation (shallow, above Moho): 0.8482
Moho RMSE: init=0.516km, inverted=0.148km

9.3 run_joint_checkerboard_sed.py — Joint Velocity–Sediment-Bottom Checkerboard Test

Function:

Joint inversion of velocity and the base of the sedimentary layer using Pg + PsedP. Three experiments compare the value of the PsedP phase and interface inversion.

Synthetic-model characteristics:

True Moho: flat at 10.5 km (not inverted in this test)

True sediment bottom: base = 5.5 km, AMP = 0.3, WAVELENGTH = 25.0 km (undulating)

Initial sediment bottom: flat at 5.5 km (to be inverted)

PsedP offset: 3–15 km (short offsets focusing on shallow sediments)

Three comparison experiments:

| Experiment | Phases | Inverted interface | Description |
| --- | --- | --- | --- |
| pg_vel | Pg | none | Baseline: Pg-only velocity inversion |
| pg_psed_vel | Pg+PsedP | none | Add PsedP but do not invert the interface |
| pg_psed_vel_sed | Pg+PsedP | iface=3 (Sed) | Joint inversion of velocity + sediment bottom |

Main outputs (example_inv_joint_sed/, with three subdirectories):

pg_vel/ — experiment 1 output

pg_psed_vel/ — experiment 2 output

pg_psed_vel_sed/ — experiment 3 output (contains iface_3.refl)

step5_comparison.pdf — three-experiment velocity comparison

step5_sed_recovery.pdf — sediment-bottom recovery comparison

step5_bar_comparison.pdf — bar-chart comparison

Key result metrics:

Overall Correlation (per experiment)

Shallow Correlation (above sed_base + 1 km)

RMS Diff (%) (per experiment)

Sed RMSE (only pg_psed_vel_sed, km)

Run example:

cd /mnt/d/FMT
python3 run_joint_checkerboard_sed.py

# Typical output:
Experiment                    Corr  Shallow     RMS%   Sed RMSE
-----------------------------------------------------------
Pg only (vel)               0.6303   0.8937    1.730        N/A
Pg+PsedP (vel)              0.2984   0.3436    2.892        N/A
Pg+PsedP (vel+sed)          0.5899   0.7970    1.784    0.095km

9.4 run_pgss_checkerboard.py — Assessment of the Pg_ss Phase Value

Function:

Evaluate the inversion value of the Pg_ss phase (source-side water-layer multiple of Pg). Four experiments compare velocity inversion using Pg, Pg_ss, Pg_ss2, and their combinations.

Four comparison experiments:

| Experiment | Phases | Description |
| --- | --- | --- |
| pg_only | Pg | Baseline |
| pg_pgss | Pg+Pg_ss | Pg + Pg_ss |
| pg_pgss2 | Pg+Pg_ss2 | Pg + Pg_ss2 |
| all | Pg+Pg_ss+Pg_ss2 | All phases |

Key inversion parameters:

SMOOTH_TYPE = 1 (Laplacian)

LAM_DAMP = 0.15, LAM_SMOOTH = 0.3

INV_NITER = 15

VEL_ONLY = True (initial Moho/Sed use true values)

Pg/Pg_ss/Pg_ss2 offsets are all 3–50 km

Main outputs (example_inv_pgss/, with four subdirectories):

Four experiment subdirectories (pg_only/, pg_pgss/, ..., all/)

step5_comparison.pdf — four-experiment velocity comparison

step5_difference.pdf — difference maps

step5_bar_comparison.pdf — bar-chart comparison

Run example:

cd /mnt/d/FMT
python3 run_pgss_checkerboard.py

# Typical output:
Experiment               Corr  Shallow     RMS%
--------------------------------------------
Pg only                0.6439   0.8855    1.695
Pg + Pg_ss             0.6478   0.8895    1.686
Pg + Pg_ss2            0.6565   0.8961    1.669
Pg + Pg_ss + Pg_ss2    0.6574   0.8983    1.666

10. Inversion Parameter Tuning Guide

10.1 Damping Coefficient (LAM_DAMP)

The damping coefficient controls the amplitude of model perturbations and balances data fitting against model complexity. L-curve analysis is recommended for selecting an optimal value.

| LAM_DAMP | Characteristic | Applicable scenario |
| --- | --- | --- |
| 0.1-0.5 | Overfitting | High-quality data, synthetic tests |
| 1.0 | Balanced | Recommended for real data (L-curve corner) |
| 2.0-5.0 | Underfitting | Noisy data, stability required |

10.2 Smoothing Coefficient (LAM_SMOOTH)

The smoothing coefficient controls model spatial smoothness:

Too small (<0.01): ray footprints remain; model is rough.

Moderate (0.02–0.15): balances resolution and stability.

Too large (>0.5): over-smoothing; details are lost.

10.3 Correlation Lengths (CORR_X / CORR_Z)

Gaussian smoothing correlation lengths control smoothing anisotropy:

CORR_X: recommended 10 km (about three grid intervals).

CORR_Z: recommended 5 km.

CORR_X_IFACE: recommended 10 km (horizontal interface smoothing).

Values that are too large over-smooth long-wavelength structures.

10.4 Smoothing-Type Selection

| Scenario | Recommended smooth_type | Reason |
| --- | --- | --- |
| Synthetic checkerboard test | 1 (Laplacian) | Preserves linear perturbations; high perturbation correlation |
| Real-data inversion | 2 (Gaussian) | Reduces ray footprints; high velocity correlation |
| Joint velocity + Moho | 3 (Hybrid) | Sharp velocity, smooth interface |
| Pg-only inversion | 2 (Gaussian) | Smooth horizontal transitions |

11. Troubleshooting

11.1 Compilation Errors

| Problem | Cause | Solution |
| --- | --- | --- |
| gfortran: command not found | Compiler not installed | Install gfortran (apt install gfortran) |
| Module dependency error | Compilation order issue | Use make, which handles dependencies automatically |
| Array-bounds SIGSEGV | Hard-coded array size too small | Check dynamic allocation and array dimensions |

11.2 Runtime Errors

| Problem | Cause | Solution |
| --- | --- | --- |
| determinant negative warning | Non-monotonic upwind stencil | Usually harmless; can be ignored |
| Ray-tracing failure | Zero traveltime gradient | Check whether the velocity model contains zero values |
| Interface-crossing failure | Non-monotonic interface depth | Check node ordering in the interface file |
| RMS does not converge | Inappropriate damping/smoothing parameters | Adjust using L-curve analysis |

11.3 Anomalous Results

| Problem | Cause | Solution |
| --- | --- | --- |
| Obvious ray footprints | Insufficient smoothing | Increase LAM_SMOOTH or switch to Gaussian smoothing |
| Interface spikes | Insufficient interface smoothing | Increase LAM_SMOOTH_IFACE or CORR_X_IFACE |
| Velocity outliers | Insufficient damping | Increase LAM_DAMP |
| Moho does not converge | Insufficient PmP data | Check PmP ray coverage and adjust REFL_WEIGHT |

— End of document —